Ede 12-3-97   version 0.6�

a Disco running light project for the 16C84 microcontroller

-----------------------------------------------------------

This project uses a single 16C84 microcontroller and some

additional hardware to make a complete programmable 8-channel

light system.

There are 16 predefined 8-stage light patterns and 4 free

programmable banks with 16 positions each to store pattern

sequences.

The system has a 8 LED status display,also used as preview/

monitor display and a 2 digit 7-segment display for speed

setting and programming purposes.

The system is controlled with 8 micro buttons.

Futher more there is a BassSync input to synchronize the

lightshow with the music.

hardware

--------

The 16C84 PORTB is used as a 8 bit data bus.there are 3

latched outputs:status/preview,2 digit 7-segment display and

output.PORTA RA0,RA2 and RA1 are used for latch control,

RA3 controls the 8 micro buttons(these are also connected

to the data bus).RA4 is the bass sync input.

An upcoming version will also provide a bass sync input

filter and an in circuit programming facility.

software

--------

The code is developed with the excellent Hi-Tech C compiler

beta.

The whole 1k program space of the 16c84 is used; you can't

add code but you can change it if you like of course.

To save RAM(register) memory BIT variables are used as

much as possible.

The array PAT[] holds the 16 predefined patterns,orginally

I planned 32 patterns but there was not enough room for that.

Most of the code will speak for it self I hope,if you have

questions ,you can always e-mail me.

use

---

the system starts up with bank 1 active and speed at 15.

buttons                              status LEDS/Display

1 output on/off                          LED1 on/off

2 monitor preview on/off                 Leds status or Monitor/Preview

3 speed up                               7-segment shows new speed

4 speed down                             7-segment shows new speed

5 bank 1                                 Led 5 on/off or flash

6 bank 2                                 Led 6 on/off or flash

7 bank 3                                 Led 7 on/off or flash

8 bank 4                                 Led 8 on/off or flash

button 3 and 4 together BassSync on/off  LED 2 on/off

button 3 and 5 together program bank 1   programming mode

button 3 and 6 together program bank 2   programming mode

button 3 and 7 together program bank 3   programming mode

button 3 and 8 together program bank 4   programming mode

you can set a bank(the LED goes on) or reset(the LED goes off),

the last set bank is the active bank.the system will loop

through all set banks.The active bank flashes.

programming mode:

buttons                                  Leds/Display

1 auto change direction on/off           Led 1 on/off

2 stop programming                       status mode

3 pattern nr. down                       7-segment shows new pattern nr.

4 pattern nr. up                         7-segment shows new pattern nr.

5 inverted on/off                        Led 4 on/off

6 nr. loops through pattern 4 times      Led 5 on/off

7 nr. loops through pattern 2 times      Led 6 on/off

8 save settings to EEPROM memory         7-segment shows memory position

With both Led 5 and 6 On ,the number of loops is 8 times,

when both are off there is only one loop.

After button 8 is activated the memory position number is

shown for about half a second,after that the pattern nr. is shown again.

you can stop programming at any point,but to program all positions

you have to proceed through all 16.

bugs ,comments

--------------

Willem Kloosterhuis

e-mail waklos@bos.nl

website www.bos.nl/homes/waklos

a double sided PCB with in circuit programming facility and BassSync

filter will be available shortly.

rev. history

0.5 first release

0.6 some changes and bugfixes

improved schematic diagram